Here is a complete, detailed document you can copy and paste into Microsoft Word. It explains your entire Snakes and Ladders circuit.

# SNAKES AND LADDERS DIGITAL CIRCUIT – COMPLETE TECHNICAL DOCUMENTATION

## Project Overview

This document describes a fully functional digital logic circuit for a two-player Snakes and Ladders game designed in LogicWorks. The game is played on a board with 32 cells (0 to 31). Players take turns rolling a dice (values 1 to 6) and move their token accordingly. The first player to reach cell 31 wins the game.

### Game Features

- Two players with alternating turns

- Dice roll generates random values between 1 and 6

- 3 Snakes:

  - Cell 25 → goes to cell 16

  - Cell 17 → goes to cell 8

  - Cell 30 → goes to cell 0

- 2 Ladders:

  - Cell 2 → goes to cell 9

  - Cell 14 → goes to cell 27

- Overshoot protection: If a dice roll would move a player beyond cell 31, the player stays in their current cell

- Winner detection: The circuit detects when a player reaches cell 31 and outputs a Winner Player (WP) signal

- Reset functionality: Returns the game to its initial state

### Inputs and Outputs

| Signal Name | Type | Description |
| --- | --- | --- |
| Clock Pulse (CP) | Input | Master clock for all sequential elements |
| Start Dice Roll | Input | 1 = enable dice roll, 0 = stop (pulse or toggle) |
| Reset | Input | Asynchronous reset – returns game to start |
| Player 1 Position | Output | 5-bit binary value (0 to 31) displayed on LEDs |
| Player 2 Position | Output | 5-bit binary value (0 to 31) displayed on LEDs |
| Winner Player (WP) | Output | 0 = Player 1 won, 1 = Player 2 won |

## Overall Circuit Architecture

The circuit is divided into eight major functional blocks:

- Position Registers (Player 1 and Player 2)

- Turn Select Multiplexer

- Dice Roll Unit

- Adder (Position + Dice)

- Overshoot Protection Comparator and Multiplexer

- Snake and Ladder Mapping Logic

- Winner Detector

- Control Logic (Turn toggling, game active)

Each block is described in detail in the following sections.

## Block 1: Position Registers (5 bits each)

### Purpose

Stores the current board position for each player.

### Implementation

Each player uses five D flip-flops (one per bit). When a valid move occurs, the new position (after dice roll, overshoot protection, and snake/ladder mapping) is loaded into the active player's register.

### Connections

- Clock: Connected to the master clock (CP) through an AND gate that enables clocking only on valid moves

- Data Input (D): Connected to the output of the Snake/Ladder Mapper

- Clear (CLR): Connected to the Reset signal (active low – when Reset = 0, positions clear to 0)

- Output (Q): Connected to:

  - 5 LEDs for visual display

  - Turn Select Multiplexer inputs

### Initial State

After Reset, both Player 1 and Player 2 positions are set to 0.

## Block 2: Turn Select Multiplexer

### Purpose

Selects which player's position is sent to the adder for the current turn.

### Implementation

Uses five 2-to-1 multiplexers (one per bit). The select line is controlled by the Turn flip-flop.

### Truth Table

| Turn FF | Selected Player |
| --- | --- |
| 0 | Player 1 position |
| 1 | Player 2 position |

### Connections

- Input 0 (I0) bits: Player 1 position (Q0–Q4)

- Input 1 (I1) bits: Player 2 position (Q0–Q4)

- Select (S): Turn flip-flop output

- Output (Y) bits: Current player position (sent to Adder)

## Block 3: Dice Roll Unit

### Purpose

Generates a random number between 1 and 6 when the Start Dice Roll input is activated.

### Implementation

Uses a 3-bit counter that increments on each clock pulse when enabled. The output is mapped to values 1 through 6:

| Counter Value | Dice Output |
| --- | --- |
| 000 (0) | 1 (001) |
| 001 (1) | 2 (010) |
| 010 (2) | 3 (011) |
| 011 (3) | 4 (100) |
| 100 (4) | 5 (101) |
| 101 (5) | 6 (110) |
| 110 (6) | 1 (001) - wraps around |
| 111 (7) | 2 (010) - wraps around |

### Connections

- Clock: Connected to master clock

- Enable: Start Dice Roll switch (must be 1 to advance counter)

- Clear: Connected to Reset

- Output (3 bits): Sent to Adder (after mapping to 1–6)

## Block 4: Adder (Position + Dice)

### Purpose

Adds the current player's position to the dice roll value.

### Implementation

Uses a 5-bit binary adder built from two 4-bit adders cascaded together (or a single 5-bit adder if available).

### Calculation

SUM = Current_Player_Position + Dice_Value

### Range

- Minimum: 0 + 1 = 1

- Maximum: 31 + 6 = 37

### Connections

- Input A (5 bits): Output of Turn Select Multiplexer (current position)

- Input B (5 bits): Dice value (1 to 6, padded with leading zeros)

- Output (5 bits): Sum (0 to 37) – sent to Overshoot Protection

## Block 5: Overshoot Protection

### Purpose

Prevents players from moving beyond cell 31. If the calculated sum exceeds 31, the player stays in their current cell.

### Implementation

Uses a 5-bit magnitude comparator and a 5-bit 2-to-1 multiplexer.

### Comparison

Compares SUM (from adder) with the constant 31 (binary 11111).

### Decision Table

| Condition | Selected Value |
| --- | --- |
| SUM ≤ 31 | SUM (move forward) |
| SUM > 31 | Current position (stay) |

### Connections

- Comparator Input A: SUM[4:0]

- Comparator Input B: 11111 (31)

- Comparator Output (A > B): Controls the multiplexer select line

- Multiplexer Input 0: SUM

- Multiplexer Input 1: Current player position (from Turn MUX output)

- Multiplexer Output: Result (sent to Snake/Ladder Mapper)

## Block 6: Snake and Ladder Mapping Logic

### Purpose

Checks if the current position is the head of a snake or ladder. If yes, it replaces the position with the corresponding tail. If not, the position remains unchanged.

### Snake and Ladder Table

| Head Position | Type | Tail Position |
| --- | --- | --- |
| 2 (00010) | Ladder | 9 (01001) |
| 14 (01110) | Ladder | 27 (11011) |
| 17 (10001) | Snake | 8 (01000) |
| 25 (11001) | Snake | 16 (10000) |
| 30 (11110) | Snake | 0 (00000) |

All other positions (0,1,3,4,5,6,7,8,9,10,11,12,13,15,16,18,19,20,21,22,23,24,26,27,28,29,31) map to themselves.

### Implementation Using Basic Gates

The circuit uses AND, OR, and NOT gates exclusively. Five detection circuits identify each head position:

#### Head Detection Equations

HEAD2  = NOT P4 AND NOT P3 AND NOT P2 AND NOT P1 AND P0
HEAD14 = NOT P4 AND P3 AND P2 AND P1 AND NOT P0
HEAD17 = P4 AND NOT P3 AND NOT P2 AND NOT P1 AND P0
HEAD25 = P4 AND P3 AND NOT P2 AND NOT P1 AND P0
HEAD30 = P4 AND P3 AND P2 AND P1 AND NOT P0

#### Combined Signals

HEAD_SUM = HEAD2 OR HEAD14 OR HEAD17 OR HEAD25 OR HEAD30
DEFAULT  = NOT HEAD_SUM

#### Output Bit Equations

N0 = HEAD2 OR HEAD14 OR (DEFAULT AND P0)
N1 = HEAD14 OR (DEFAULT AND P1)
N2 = DEFAULT AND P2
N3 = HEAD_SUM OR (DEFAULT AND P3)
N4 = (HEAD14 OR HEAD17 OR HEAD25 OR HEAD30) OR (DEFAULT AND P4)

Where:

- P4–P0 = Input position (from Overshoot Protection)

- N4–N0 = Output position (to Position Registers)

### Gate Count

| Gate Type | Quantity |
| --- | --- |
| NOT | 6 |
| AND (2-input) | 16 |
| OR (2-input) | 12 |
| Total | 34 gates |

### Testing the Snake/Ladder Mapper

| Input (Binary) | Decimal | Expected Output (Binary) | Decimal |
| --- | --- | --- | --- |
| 00010 | 2 | 01001 | 9 |
| 01110 | 14 | 11011 | 27 |
| 10001 | 17 | 01000 | 8 |
| 11001 | 25 | 10000 | 16 |
| 11110 | 30 | 00000 | 0 |
| 00101 | 5 | 00101 | 5 |
| 11111 | 31 | 11111 | 31 |

## Block 7: Winner Detector

### Purpose

Detects when a player reaches cell 31, latches the win, and outputs which player won.

### Implementation

#### Step 1: Detect Position = 31

POS_IS_31 = P4 AND P3 AND P2 AND P1 AND P0

Uses four AND gates in a tree configuration.

#### Step 2: Win Latch

A D flip-flop stores the win state:

- D input: POS_IS_31 AND GAME_ACTIVE

- CLK: Position update clock

- CLR: Reset signal

- Q output: WIN (1 = game over)

#### Step 3: Game Active Signal

GAME_ACTIVE = NOT WIN

This signal disables dice rolls when the game is over.

#### Step 4: Winner Player Output

WP = WIN AND TURN

- If Player 1 wins (TURN = 0) → WP = 0

- If Player 2 wins (TURN = 1) → WP = 1

### Truth Table

| Current Position | TURN | GAME_ACTIVE | POS_IS_31 | Next WIN | WP |
| --- | --- | --- | --- | --- | --- |
| 31 | 0 | 1 | 1 | 1 | 0 |
| 31 | 1 | 1 | 1 | 1 | 1 |
| Any other | Any | 1 | 0 | 0 (or unchanged) | — |
| Any (after win) | Any | 0 | X | 1 | Stays |

## Block 8: Control Logic

### Purpose

Manages turn switching and game flow.

### Turn Flip-Flop

A D flip-flop stores the current turn:

- Q = 0: Player 1's turn

- Q = 1: Player 2's turn

### Turn Toggling

After each valid move (when a dice roll completes and the position updates), the turn flip-flop toggles:

TURN_next = TURN_current XOR UPDATE_PULSE

Where UPDATE_PULSE is generated when:

- A dice roll occurred

- The game is still active (no winner)

- The position update is complete

### Game Flow States

| State | Description |
| --- | --- |
| RESET | Game starts; P1=0, P2=0, TURN=0, WIN=0 |
| WAIT | Waiting for Start Dice Roll = 1 |
| ROLL | Dice counter increments |
| UPDATE | Position is updated (adder + protection + snakes) |
| CHECK WIN | If position = 31, set WIN and WP |
| TOGGLE TURN | Switch to other player |
| GAME OVER | If WIN=1, ignore all dice rolls until Reset |

## Complete Signal Flow Diagram (Text View)

┌─────────────────────────────────────────────────────────────────────────────┐
                    │                           COMPLETE CIRCUIT DATAFLOW                         │
                    └─────────────────────────────────────────────────────────────────────────────┘

RESET ──────────────────────────────────────────────────────────────────────────────────────────┐
                                                                                                 │
CLOCK ──┬─────────────────────────────────────────────────────────────────────────────────────┐ │
        │                                                                                     │ │
        ▼                                                                                     │ │
┌───────────────┐     ┌──────────────┐     ┌──────────────┐     ┌──────────────┐              │ │
│   Dice Roll   │     │ Turn MUX     │     │    Adder     │     │ Overshoot    │              │ │
│   (1-6)       │────▶│ Selects      │────▶│ Pos + Dice   │────▶│ >31? MUX     │              │ │
└───────────────┘     │ active player│     └──────────────┘     └──────┬───────┘              │ │
                      └──────┬───────┘                                 │                      │ │
                             │                                         │                      │ │
                      ┌──────┴───────┐                                │                      │ │
                      │ Player 1 Pos │                                │                      │ │
                      │ Player 2 Pos │◄───────────────────────────────┘                      │ │
                      └──────────────┘                                                        │ │
                             │                                                               │ │
                             ▼                                                               │ │
                      ┌──────────────┐     ┌──────────────┐                                  │ │
                      │ Snake/Ladder │────▶│ Winner Det.  │────▶ WP                          │ │
                      │   Mapper     │     │ (Pos = 31?)  │                                  │ │
                      └──────────────┘     └──────┬───────┘                                  │ │
                                                  │                                          │ │
                                                  ▼                                          │ │
                                           ┌──────────────┐                                 │ │
                                           │ Turn Toggle  │◄────────────────────────────────┘ │
                                           │ & Game Active│                                   │
                                           └──────────────┘                                   │
                                                                                              │
                                      ┌───────────────────────────────────────────────────────┘
                                      │
                                      ▼
                              (Feedback to Dice Enable)

## Timing and Clocking

### Clock Signal

The master clock (CP) drives all sequential elements. For simulation, a 1 Hz clock or manual pulse button can be used.

### Sequence of Events per Turn

- Player presses Start Dice Roll (sets enable = 1)

- Clock edge increments dice counter (value changes 1–6)

- Adder computes Position + Dice

- Overshoot comparator checks if result > 31

- Multiplexer selects either sum or current position

- Snake/Ladder mapper checks for heads and replaces with tails if needed

- New position is loaded into active player's register on next clock edge

- Winner detector checks if new position = 31

  - If yes: WIN = 1, WP is set, game stops

  - If no: Turn flip-flop toggles, next player's turn begins

## Reset Behavior

When the Reset button is pressed (active low, meaning Reset = 0 to clear):

| Component | Result |
| --- | --- |
| Player 1 Position Register | Clears to 0 |
| Player 2 Position Register | Clears to 0 |
| Turn Flip-Flop | Clears to 0 (Player 1's turn) |
| Winner Latch | Clears to 0 (WIN = 0) |
| Dice Counter | Clears to 0 |
| Game Active | Becomes 1 |

After Reset is released (Reset = 1), the game is ready to begin with Player 1 at cell 0.

## Testing and Validation

### Test Case 1: Normal Move (No Snake/Ladder)

- Player 1 at cell 5, rolls 3 → moves to cell 8

- Expected: Position = 8, no win

### Test Case 2: Ladder (2 → 9)

- Player at cell 2, rolls any value after landing on 2 → moves to 9

- Expected: Position jumps from 2 to 9

### Test Case 3: Snake (25 → 16)

- Player at cell 25, rolls any value after landing on 25 → moves to 16

- Expected: Position drops from 25 to 16

### Test Case 4: Overshoot Protection

- Player at cell 30, rolls 5 → 30+5=35 (>31) → stays at 30

- Expected: Position unchanged at 30

### Test Case 5: Win Condition

- Player at cell 26, rolls 5 → 26+5=31 → winner

- Expected: WIN = 1, WP = current player (0 or 1), game stops accepting dice rolls

### Test Case 6: Reset

- After any player wins, press Reset

- Expected: Both players at 0, Turn = Player 1, WIN = 0, dice roll enabled again

## Complete Gate and Component Summary

### Sequential Components

| Component | Quantity | Purpose |
| --- | --- | --- |
| D Flip-Flop | 11 | Position registers (2×5 bits), Turn FF, Winner latch |
| 3-bit Counter | 1 | Dice roll generation |

### Combinational Components

| Component | Quantity | Purpose |
| --- | --- | --- |
| 2-to-1 MUX (5-bit) | 2 | Turn selection, overshoot protection |
| 5-bit Adder | 1 | Position + Dice |
| 5-bit Comparator | 1 | Overshoot detection (>31) |
| 5-bit Comparator | 1 | Win detection (=31) |

### Gates for Snake/Ladder Mapper

| Gate Type | Quantity |
| --- | --- |
| NOT | 6 |
| AND | 16 |
| OR | 12 |

### Total Gate Equivalent Count

Approximately 100–120 gate equivalents, not including the adder and comparators (which are built from gates internally in LogicWorks).

## Troubleshooting Guide

| Symptom | Possible Cause | Solution |
| --- | --- | --- |
| Player moves beyond 31 | Overshoot comparator not connected | Check comparator and MUX wiring |
| Snake/ladder not working | Head detection incorrect | Test each HEAD signal with known inputs |
| Turn doesn't switch | Missing UPDATE_PULSE signal | Check turn toggle logic connection |
| Win doesn't latch | D flip-flop CLR or CLK issue | Verify DFF connections |
| Both players move simultaneously | Position register clocks not gated | Add AND gate with GAME_ACTIVE and turn |
| Dice always shows 0 | Counter not enabled or no clock | Check Start Dice Roll and clock connection |
| Reset doesn't clear everything | Some FFs missing CLR connection | Connect Reset to all CLR pins |

## Conclusion

This circuit implements a complete, functional Snakes and Ladders game using standard digital logic components available in LogicWorks. The design carefully avoids any "don't care" states by ensuring all combinational outputs are defined for every possible input combination and all flip-flops have explicit reset values.

The modular design allows for easy testing and debugging of each block independently. The use of basic gates for the snake/ladder mapper ensures compatibility with LogicWorks versions that may not have ROM or complex IC components.

Final Status: ✅ Complete and ready for simulation

## Appendix: Pin Naming Convention

Throughout this document, the following naming conventions are used:

- P0 = Least Significant Bit (LSB) of position

- P4 = Most Significant Bit (MSB) of position

- N0 = Next position LSB (after snake/ladder)

- N4 = Next position MSB

- WP = Winner Player output

- CP = Clock Pulse

- FF = Flip-Flop

- MUX = Multiplexer

Document Version: 1.0

Date: [Current Date]

Software: LogicWorks

Game Version: Snakes and Ladders (32 cells, 3 snakes, 2 ladders)

You can now copy everything above and paste it directly into Microsoft Word. The formatting will preserve the headings, tables, and ASCII diagrams. If you want me to adjust anything (add more diagrams, simplify sections, or add a parts list table for ordering components), just let me know.